CO2 PROBLEM SOLVING ASSIGNMENT

# Abstract:

This report develops a practical transmitter for an eight-channel dense wavelength-division multiplexed (DWDM) link at 1550 nm. Each wavelength carries 25 Gb/s NRZ over six amplified 80 km fiber spans. Instead of choosing a source based only on its output power, the design checks emission wavelength, quantum efficiency, modulation speed, temperature drift, nonlinear effects, soliton conditions and integration options. The calculations show that a wavelength-locked DFB laser with external modulation is a sensible starting point, but several requirements still need laboratory and standards verification before deployment.

# Interpret the communication requirement :

Eight 25 Gb/s channels provide 200 Gb/s aggregate raw line rate. Their nominal spacing is 100 GHz, or about 0.8 nm near 1550 nm. Six 80 km spans give a 480 km optical path. Long-distance transmission needs narrow-linewidth, single-frequency lasers to prevent adjacent-channel interference. The transmitter also needs adequate optical power, low chirp, controlled wavelength and reliable high-speed modulation. Amplifiers restore power after each span but do not undo accumulated distortion or amplified spontaneous-emission noise.

Bandgap sets the approximate emission photon energy; internal quantum efficiency describes radiative versus total recombination.

Threshold current and slope efficiency determine useful output above lasing threshold; linewidth and side-mode suppression ratio (SMSR) affect channel isolation.

Laser chirp, chromatic dispersion, relative-intensity noise, extinction ratio and temperature drift influence the received eye opening.

Fiber loss, effective length, nonlinear coefficient, dispersion, channel spacing and launch power determine nonlinear interactions.

ITU-T G.957 concerns optical interfaces for SDH systems; IEEE 802.3bm and 802.3bs concern specified Ethernet PHY interfaces. Neither automatically certifies this custom 8×25 Gb/s 480 km DWDM line.

Eye safety must be assessed against the applicable ANSI Z136 exposure and laser-classification framework, with wavelength, exposure duration, aperture and fault conditions defined.

Problem requirement statement: Build eight stable 1550 nm optical channels, each carrying 25 Gb/s NRZ, with sufficient optical power, at least 17.5 GHz nominal electrical modulation bandwidth, controlled chirp and wavelength drift, acceptable nonlinear penalties, safe accessible emissions and a scalable transmitter architecture.

Emission wavelength and internal efficiency:

Photon emission occurs when electrons and holes recombine radiatively; non-radiative recombination wastes carrier energy as heat. Given Eg = 0.80 eV: λ (µm) ≈ 1.24/Eg = 1.24/0.80 = 1.55 µm = 1550 nm.

Given radiative lifetime τr = 10 ns and non-radiative lifetime τnr = 40 ns: ηint = (1/τr)/[(1/τr)+(1/τnr)] = τnr/(τr+τnr) = 40/(10+40) = 0.80 = 80%. This is the radiative recombination fraction in the simplified lifetime model, not the complete wall-plug efficiency.

## Source comparison:

| Source | Spectrum / speed | Power, threshold and coupling | Assessment |
| --- | --- | --- | --- |
| LED | Broad spectrum; limited direct modulation speed | No lasing threshold; weak coupling to single-mode fiber | Unsuitable for 100 GHz long-haul DWDM |
| FP laser | Multiple longitudinal modes; fast variants exist | Lasing threshold; good guided coupling | Mode competition makes dense channels difficult |
| DBR laser | Narrow spectrum; tunable variants | Good power and fiber coupling; temperature/current tuning | Strong alternative when tunability is important |
| DFB laser | Single longitudinal mode; narrow linewidth; fast | Stable wavelength with feedback; strong single-mode coupling | Preferred source for fixed-grid channels |
| VCSEL | Compact and efficient; fast variants | Low threshold; convenient arrays; 1550 nm options need qualification | Interesting for short reach or qualified 1550 nm arrays |
| QD laser | Potential low chirp and temperature sensitivity | Performance varies greatly with platform and maturity | Promising alternative after device-level validation |

Laser oscillation begins when round-trip optical gain equals round-trip cavity loss. For a DWDM source, high SMSR suppresses unwanted longitudinal modes; low linewidth limits spectral overlap and phase-noise penalties. Exact SMSR and linewidth targets must be set from the modulation format and receiver design, not invented from the given data.

Design Decision 1: I recommend a 1550 nm DFB laser for each fixed DWDM channel because it supports narrow-linewidth, single-mode emission, good fiber coupling and stable wavelength locking. A tunable DBR laser is a credible alternative when remote channel reconfiguration or wavelength inventory reduction is required.

# Modulation, chirp and wavelength stability

## Laser output and NRZ bandwidth

Ith = 12 mA, I = 40 mA, ηs = 0.3 mW/mA. P = ηs(I − Ith) = 0.3 × (40 − 12) = 8.4 mW. In dBm: P(dBm) = 10 log10(8.4/1) = 9.24 dBm. This is the estimated laser output before modulator and coupling losses, not the 2 mW per-channel fiber launch.

Required NRZ bandwidth B ≈ 0.7Rb = 0.7 × 25 GHz = 17.5 GHz. Given 3 dB bandwidth = 20 GHz; margin = 20 − 17.5 = 2.5 GHz (approximately 14.3% above the required bandwidth). The simplified bandwidth test passes, although eye-mask and rise-time tests are still necessary.

## Rate equations and relaxation oscillation:

A standard simplified laser model is dN/dt = I/(qV) − N/τn − G(N,S)S and dS/dt = ΓG(N,S)S + βsp N/τr − S/τp, where N is carrier density, S is photon density, Γ is confinement factor, τp is photon lifetime and G is optical gain. The relaxation-oscillation frequency is associated with the coupled carrier–photon response and often increases approximately with the square root of optical power above threshold. Small-signal modulation is a linearized response around bias; large-signal modulation introduces pattern effects, turn-on delay, overshoot and stronger chirp. A numeric relaxation frequency cannot be determined without gain and photon-lifetime parameters.

## Direct versus external modulation:

| Method | Benefit | Limitation / decision |
| --- | --- | --- |
| Direct laser modulation | Low component count and potentially low cost | Carrier-driven wavelength chirp can interact with chromatic dispersion over 480 km |
| External Mach–Zehnder modulation | Lower transmitter chirp when properly biased; good high-speed optical quality | Additional insertion loss, driver power and bias-control complexity |

Uncontrolled drift: Δλ = (0.1 nm/°C)(40 °C) = 4.0 nm, equivalent to about five 0.8 nm channel spacings. To hold the drift within ±0.1 nm, |ΔT| ≤ 0.1/0.1 = ±1 °C relative to the set point, assuming the stated coefficient and ignoring ageing. A TEC with thermistor feedback and wavelength locker is preferred. Injection-current tuning is useful for fine trimming but changes optical power, chirp and bias; it should not replace thermal stabilization.

Design Decision 2: I recommend external Mach–Zehnder modulation and closed-loop TEC plus wavelength-locker control because 20 GHz nominal bandwidth exceeds 17.5 GHz, external modulation reduces chirp risk, and uncontrolled 4 nm drift is far too large for 0.8 nm spacing. An output-power controller must account for modulator insertion loss so the fiber launch remains at the intended 2 mW.

Nonlinearity and eye safety

## Effective length and self-phase modulation

Convert attenuation to nepers per km for the power equation: α = (ln10/10) × 0.2 = 0.04605 km⁻¹. For L = 80 km: Leff = [1 − exp(−0.04605 × 80)]/0.04605 = 21.17 km. (Using 0.2 directly as km⁻¹ would be incorrect because the input is in dB/km.)

With γ = 1.3 W⁻¹km⁻¹ and P = 2 mW = 0.002 W: φNL,span = γPLeff = 1.3 × 0.002 × 21.17 = 0.0550 rad. Approximate six-span total = 6 × 0.0550 = 0.330 rad if each span is restored to the same launch power. This simplified estimate excludes channel-dependent amplification, pulse peaks and inter-channel effects.

## XPM, FWM, SBS and SRS:

XPM arises when power fluctuations in neighboring WDM channels change the refractive index seen by a channel. FWM creates new frequency components from combinations of channel frequencies and can cause crosstalk if products fall on DWDM channels. XPM and FWM increase with launch power; dispersion and channel spacing influence phase matching and temporal walk-off. Standard dispersive fiber usually reduces FWM efficiency compared with near-zero-dispersion fiber.

SBS threshold: Aeff = 80 µm² = 80 × 10⁻¹² m²; Leff = 21.17 km = 21170 m. Pth,SBS ≈ 21Aeff/(gBLeff) = 21(80×10⁻¹²)/[(5×10⁻¹¹)(21170)] = 0.00159 W ≈ 1.59 mW.SRS threshold:Pth,SRS ≈ 16Aeff/(gRLeff) = 16(80×10⁻¹²)/[(6×10⁻¹⁴)(21170)] = 1.008 W ≈ 1008 mW.

At 2 mW/channel, the stated narrow-line SBS estimate is below launch power, indicating SBS needs careful checking. The total eight-channel launch is 8 × 2 = 16 mW = 12.04 dBm, well below the simple SRS threshold estimate. The SBS threshold equation is a narrow-line, effectively continuous-wave approximation; high-speed data modulation and optical linewidth broadening can substantially increase the practical SBS threshold. Total WDM power should not simply be compared with a single-tone SBS threshold.

| Effect | Origin / behavior | Mitigation |
| --- | --- | --- |
| SPM | A channel changes its own optical phase | Moderate launch power, dispersion-aware DSP |
| XPM | Neighboring channels impose phase changes | Optimize launch power, dispersion map and channel allocation |
| FWM | Kerr mixing generates new frequencies | Avoid low-dispersion phase matching; optimize grid and power |
| SBS | Narrowband backward Brillouin scattering | Linewidth broadening when compatible, monitoring and launch optimization |
| SRS | Broadband Raman power transfer, often shorter-to-longer wavelengths | Power tilt control, gain equalization and total-power management |

Eye-safety assessment: 16 mW is the sum of guided optical powers, not an accessible emission classification. ANSI Z136 permissible exposure depends on wavelength, exposure duration, beam geometry and measurement aperture; there is no defensible universal 16 mW pass/fail limit. As an illustrative conservative engineering control, require the connector-accessible emission to meet the applicable product classification (often assessed under IEC 60825-1) with automatic shutdown on open connectors. Formal ANSI Z136 evaluation is pending; do not claim compliance from total guided power alone.

Design Decision 3: I recommend controlled per-channel launch power, narrow optical filtering and gain equalization, plus an SBS-specific linewidth/return-loss check and automatic laser shutdown. This retains useful signal power while addressing a 0.330 rad simplified six-span SPM phase and an SBS threshold estimate near the nominal 2 mW channel power.

# Solitons, DSP, standards and integration

## Soliton calculations

An optical soliton is possible when anomalous group-velocity dispersion and Kerr self-phase modulation balance for an appropriate pulse shape and peak power. Given T0 = 15 ps and |β2| = 20 ps²/km: LD = T0²/|β2| = 225/20 = 11.25 km.

Fundamental soliton requires LNL = LD. P0,soliton = 1/(γLD) = 1/(1.3 × 11.25) = 0.0684 W = 68.4 mW peak.

At the stated P0 = 2 mW: LNL = 1/(γP0) = 1/(1.3 × 0.002) = 384.6 km. N² = LD/LNL = 11.25/384.6 = 0.02925; N = 0.171. Since N is much less than 1, the given power and pulse do not support a fundamental soliton. NRZ data also do not naturally provide the isolated sech-shaped pulses assumed by this simple soliton model.

## Digital backpropagation:

Digital backpropagation (DBP) numerically reverses the estimated dispersive and Kerr propagation of fiber, typically using split-step Fourier processing after coherent reception. It is computationally demanding and needs accurate channel and span information. Full-field compensation of all eight WDM channels is particularly difficult because it requires sufficiently broad joint-bandwidth observation. For the specified 25 Gb/s NRZ system, conventional chromatic-dispersion management and receiver equalization are the first practical options. Single-channel DBP may be evaluated only if coherent receiver architecture and measured nonlinear penalties justify it.

Standards applicability

| Document | Relevant checks | Scope caution |
| --- | --- | --- |
| ITU-T G.957 | Optical interface application code, wavelength, launch power, spectral characteristics and receiver parameters where applicable | An SDH optical interface recommendation, not a direct specification for arbitrary 8-channel 480 km DWDM |
| IEEE 802.3bm / 802.3bs | Relevant 100G/200G/400G Ethernet PHY PMD and optical interface requirements where a chosen PHY is used | Eight independent 25 Gb/s DWDM wavelengths do not automatically implement a compliant Ethernet PHY |
| ANSI Z136 / IEC 60825-1 | Accessible exposure, laser classification, labeling, interlocks and failure modes | Compliance requires wavelength-specific testing and actual accessible-emission measurements |

## Integration options:

| Architecture | Advantages | Trade-offs |
| --- | --- | --- |
| Discrete DFB + external modulator | Mature, individually replaceable, straightforward characterization | Larger footprint, more optical connectors and alignment |
| Monolithic InP | Laser and active optical functions on one platform; compact multi-channel design | Process complexity, thermal crosstalk and fabrication yield |
| Si photonics + hybrid InP laser | Scalable passive routing and modulators with integrated electronics potential | Laser attachment, packaging, coupling loss and thermal engineering |

Design Decision 4: I recommend a discrete DFB-plus-external-modulator design for the first verified 8-channel prototype, with a later migration to hybrid InP/Si photonics if cost and packaging targets justify it. I do not recommend soliton transmission at 2 mW, nor full-band DBP as a default requirement. Validate dispersion compensation and nonlinear penalties experimentally before adding complex DSP.

# Verification:

| Requirement | Target / calculated | Status |
| --- | --- | --- |
| Source | 1550 nm DFB; ηint = 80% | Conceptually suitable; linewidth/SMSR must be specified |
| Modulation and power | 17.5 GHz needed; 20 GHz available; 8.4 mW raw laser output | Pass on simplified bandwidth; optical launch after modulator to be verified |
| Wavelength/chirp | 4 nm drift without control; TEC target ±1 °C | Needs wavelength-lock and eye-diagram testing |
| Nonlinearity and safety | Leff 21.17 km; φ6span 0.330 rad; SBS 1.59 mW; SRS 1.008 W | SBS and accessible-emission tests outstanding |
| Soliton / dispersion | LD 11.25 km; N = 0.171 | Soliton not feasible at given power |
| Integration / DSP / standards | Discrete prototype; dispersion compensation; applicable interface and safety tests | Conditional; not yet certified |

# Final transmitter design:

| Parameter | Adopted design |
| --- | --- |
| Channels / grid | 8 channels, 100 GHz nominal spacing around 1550 nm |
| Data rate / distance | 25 Gb/s NRZ/channel, 6 × 80 km = 480 km |
| Laser | DFB, nominal 1550 nm; eight wavelength-set devices |
| Laser operating point | Ith 12 mA; bias 40 mA; estimated pre-modulator output 8.4 mW (9.24 dBm) |
| Modulation | External Mach–Zehnder modulator; 20 GHz candidate 3 dB bandwidth |
| Wavelength control | TEC + wavelength locker; temperature error within ±1 °C by simple drift criterion |
| Fiber launch | 2 mW per channel (3.01 dBm); 16 mW aggregate |
| Nonlinear controls | Power optimization, optical filtering, SBS evaluation, gain equalization |
| Dispersion / DSP | Dispersion compensation/equalization; no default full-band DBP |
| Integration | Discrete initial design; hybrid Si-photonics as scale-up option |
| Safety / standards | ANSI Z136 risk assessment, IEC laser classification, selected interface-standard testing |

The eight wavelength-locked DFB lasers feed external modulators, optical power monitors and a 100 GHz multiplexer. The combined signal passes through six amplified fiber spans. Feedback maintains wavelength and power while a safety controller disables emission during faults. A receiver-side dispersion/equalization plan must be included in the complete link budget.

# Final engineering decision:

Decision: NO for immediate field deployment; YES for controlled prototype development. The source wavelength, efficiency, power and bandwidth calculations support the proposed architecture, and the soliton result correctly rules out a simple soliton solution. However, the narrow-line SBS estimate is close to the intended launch power, temperature stability must be demonstrated, and actual eye-safety and interface compliance have not been measured. Before release, the team must complete a 480 km OSNR/BER link budget, modulator eye-mask and chirp tests, SBS/FWM/XPM characterization, wavelength-lock cycling, accessible-emission certification and reliability testing.

# Conclusion:

A DFB laser with external modulation and closed-loop wavelength control is the most defensible starting design for this eight-channel DWDM link. The calculations establish 1550 nm emission, 80% simplified internal efficiency, 8.4 mW estimated laser output, 2.5 GHz nominal modulation margin, 4 nm uncontrolled thermal drift, 0.330 rad approximate accumulated SPM phase, and a soliton order of 0.171. The engineering conclusion is conditional because a workable optical design still needs verified noise, dispersion, nonlinear, safety and standards performance.

# References:

G. P. Agrawal, Fiber-Optic Communication Systems, 4th ed., Wiley, 2023.

G. P. Agrawal, Nonlinear Fiber Optics, 6th ed., Academic Press, 2025.

ITU-T Recommendation G.957, Optical interfaces for equipments and systems relating to the synchronous digital hierarchy.

IEEE Std 802.3bm and IEEE Std 802.3bs, Ethernet optical PHY amendments; consult the exact selected PHY clauses.

ANSI Z136.1, Safe Use of Lasers; IEC 60825-1, Safety of laser products. Check current editions and local applicability.