ECA18 – OPTOELECTRONICS AND OPTICAL COMMUNICATION

CO4 – INVESTIGATIVE ASSIGNMENT

ULTRA-HIGH-SPEED PHOTONIC TRANSPORT LINK

Abstract:

This report investigates an ultra-high-speed Dense Wavelength Division Multiplexing (DWDM) transport link designed to carry 32 wavelength channels at 200 Gb/s per channel. The resulting nominal aggregate capacity is 6.4 Tb/s. The design study compares intensity-modulation formats with coherent modulation, evaluates photonic modulator structures, and examines optical amplification and digital signal processing (DSP) options. The proposed baseline uses dual-polarization 16-QAM (DP-16QAM) with a coherent receiver because it offers a practical balance between spectral efficiency and receiver-side impairment correction. A silicon-photonic Mach–Zehnder modulator is selected as the compact integration option, subject to confirmation of its measured electro-optic bandwidth, drive voltage and insertion loss.

For amplification, the report proposes a hybrid distributed Raman and EDFA arrangement. Raman gain can improve the span noise performance, while EDFAs provide practical lumped gain and output power. Gain-flattening and per-channel power equalization are included to control WDM power tilt. At the receiver, the recommended DSP sequence includes timing recovery, chromatic-dispersion compensation, adaptive polarization demultiplexing, carrier-frequency and phase recovery, symbol decisions and forward-error-correction decoding. Digital backpropagation is considered an optional enhancement where nonlinear penalties justify its processing cost.

The numerical work establishes the aggregate capacity, an illustrative baud-rate comparison, channel-grid assumptions and amplifier power calculations using clearly stated engineering assumptions because the assignment does not specify the input and output power per channel. The report also provides a proposed system block diagram, parameter table, verification matrix and simulation plan. No simulated BER, Q-factor or eye-diagram result is claimed as measured; these values must be obtained from OptiSystem, OptSim or MATLAB before a final field deployment. The resulting recommendation is a conditional engineering ‘yes’: the architecture is suitable for prototype validation, followed by standards and performance testing.

Keywords: DWDM, DP-16QAM, coherent optics, EDFA, Raman amplification, optical DSP

# Introduction and Engineering Requirement:

Modern cloud services, AI workloads, video platforms and data-centre interconnection continue to increase the demand for high-capacity optical transport. DWDM meets this demand by carrying many independently modulated optical channels through the same fibre. Each channel uses a different optical carrier frequency, and a multiplexer combines the carriers before transmission.

The assigned design target is 32 DWDM channels, each carrying 200 Gb/s. The system must maintain useful optical signal-to-noise ratio (OSNR), low bit-error rate (BER), acceptable Q-factor, manageable nonlinear distortion and scalable photonic integration. The investigation must select a modulation format, modulator, amplifier arrangement and DSP strategy rather than simply describe alternatives.

## Problem statement:

Design a technically defensible 32-channel DWDM link with a nominal aggregate throughput of 6.4 Tb/s. The design should balance spectral efficiency, optical reach, OSNR margin, hardware complexity, power consumption and deployment cost. All numerical results in this report are analytical estimates based on the assumptions stated alongside them.

## Design assumptions and scope:

| Parameter | Design assumption / note |
| --- | --- |
| Number of channels | 32 |
| Net target per channel | 200 Gb/s; framing and FEC overhead are not specified |
| Aggregate nominal capacity | 6.4 Tb/s before any unspecified overhead |
| Wavelength band | C-band, approximately 1530–1565 nm |
| Channel grid | 50 GHz nominal grid assumed for planning; confirm exact frequency slots and interface against the applicable ITU-T recommendation |
| Fibre | Standard single-mode fibre, representative SMF-28-type parameters |
| Amplifier calculation | Illustrative input −10 dBm/channel and target output +0 dBm/channel; these are assumptions because the brief leaves the levels blank |
| Modulation baseline | DP-16QAM, coherent detection |
| Validation | Analytical estimates plus proposed OptiSystem / OptSim / MATLAB verification; no unrun simulation is presented as a result |

# Literature and Technology Review:

The main engineering principle is that no single modulation format or amplifier is best for every link. Lower-order formats generally tolerate poorer OSNR and are simpler, while higher-order formats carry more bits per symbol but need cleaner signals and more precise transmitter and receiver hardware. Coherent detection measures amplitude and phase and can use DSP to correct dispersion and polarization effects. This makes it suitable for high-capacity channels where spectral efficiency is important.

ITU-T G.698-series recommendations address optical interfaces for multi-channel systems, including defined optical parameters for applications. IEEE 802.3 standards define particular Ethernet physical-layer interfaces rather than approving every possible DWDM design. OIF Common Electrical I/O (CEI) implementation agreements concern high-speed electrical interfaces between components. Therefore, compliance should be checked against the exact interface variant and its optical/electrical limits; it should not be claimed solely because a system uses DWDM.

| Technology | Strengths | Limitations / design implication |
| --- | --- | --- |
| NRZ-OOK / RZ-OOK | Simple transmitters and direct detection; relatively low DSP burden | Low spectral efficiency for 200 Gb/s; high bandwidth and reach constraints make it a weak baseline here |
| PAM4 | Two bits per symbol; lower symbol rate than binary signalling for the same gross bit rate | Amplitude levels are closer together, increasing sensitivity to noise, linearity and level distortion |
| DPSK / DQPSK | Differential phase coding; useful sensitivity and bandwidth trade-offs in suitable systems | Typically less flexible than modern coherent QAM for DSP-based long-haul compensation |
| 16-QAM | Four bits per symbol; higher spectral efficiency than QPSK | Needs higher OSNR and accurate phase/amplitude control |
| DP-16QAM | Four bits per symbol per polarization; coherent DSP and polarization multiplexing provide high capacity per carrier | Requires coherent optics, high-speed ADC/DACs and complex DSP |
| EDFA | Mature C-band gain, high output power, practical deployment | ASE noise accumulates over cascaded spans; gain tilt requires control |
| Raman amplifier | Distributed gain can improve effective noise performance and span OSNR | Pump lasers and safety/control complexity; design is fibre- and span-dependent |
| Hybrid Raman + EDFA | Combines distributed gain with practical lumped output gain | More components and control complexity than EDFA-only |
| Silicon photonics | Compact integration and potential for scalable manufacturing | Thermal tuning and optical loss must be managed |
| Lithium niobate | Strong electro-optic response and high-speed modulation capability | May require a larger device footprint depending on implementation |
| EO polymer | Potentially high electro-optic efficiency and compact devices | Long-term reliability, packaging and manufacturing qualification must be checked |

# Interpret the Communication Requirement:

## Aggregate capacity

Aggregate capacity = number of channels × data rate per channel.

= 32 × 200 Gb/s = 6,400 Gb/s = 6.4 Tb/s.

This is the nominal line-rate aggregate. If 200 Gb/s is a client payload rate rather than the optical line rate, FEC and framing overhead would require a higher line rate; the brief does not specify that distinction.

## Key performance metrics

| Metric | Meaning and purpose |
| --- | --- |
| Spectral efficiency (b/s/Hz) | Payload or line bit rate divided by occupied bandwidth; indicates how efficiently the spectrum is used |
| OSNR | Optical signal power relative to optical noise in a stated reference bandwidth; must exceed the selected format's receiver requirement with margin |
| BER | Fraction of received bits that are wrong; assess pre-FEC and post-FEC values separately |
| Q-factor | Signal-quality measure related to the separation of symbol distributions; the simulator's definition must be stated |
| Launch power | Power per channel at the fibre input; too low worsens noise-limited performance, while too high increases nonlinear penalties |
| Insertion loss | Power lost through modulators, couplers, filters and connectors |
| Modulation bandwidth | Electro-optic bandwidth needed to support the chosen baud rate and pulse shaping |
| Thermal / footprint limits | Packaging, cooling, tuning and rack or module area constraints |

## Impairments and matching countermeasures

| Impairment | Main effect | Mitigation |
| --- | --- | --- |
| Chromatic dispersion (CD) | Different spectral components travel at different speeds; pulses spread | Coherent DSP frequency-domain or FIR equalization |
| Polarization-mode dispersion (PMD) | Polarization components experience different delays | Adaptive MIMO polarization equalizer |
| SPM | A channel's intensity changes its own optical phase through Kerr nonlinearity | Set launch power carefully; optional DBP |
| XPM | Power variations in one channel affect the phase of neighbouring channels | Power equalization, suitable channel spacing and optional DBP |
| FWM | Multiple carriers mix and generate new frequency components | Appropriate grid/power planning and fibre-aware design |
| ASE noise | Amplifiers add broadband spontaneous-emission noise | Low-noise amplification, fewer unnecessary spans, OSNR budgeting |
| WDM crosstalk / filter leakage | Imperfect mux/demux isolation contaminates adjacent channels | AWG/MMI filter design, passband alignment and isolation testing |
| PDL | Different polarization states experience different loss | Low-PDL components and receiver equalization |

## Investigation requirement statement:

The proposed system shall carry 32 DWDM channels with a nominal rate of 200 Gb/s per channel and 6.4 Tb/s aggregate capacity. It shall use a coherent modulation and receiver architecture that supports DSP compensation, keep each channel within its assigned frequency slot, maintain sufficient OSNR above the selected pre-FEC threshold, control channel power and gain tilt, and demonstrate acceptable BER/Q-factor over the intended fibre span. The final design must be validated using the exact applicable standards, component specifications and simulation or laboratory measurements.

# Modulation and Modulator Selection:

## Candidate modulation formats

| Format | Bits per symbol / polarization | Illustrative symbol-rate relation for 200 Gb/s | Assessment |
| --- | --- | --- | --- |
| NRZ-OOK | 1 bit/symbol | ≈200 GBd before overhead | Simple but very high baud rate for this target |
| PAM4 | 2 bits/symbol | ≈100 GBd for one intensity stream, before overhead | Lower baud than OOK; direct-detection SNR and linearity are challenging |
| DPSK | 1 bit/symbol in a basic binary form | ≈200 GBd for 200 Gb/s | Differential phase benefits but not the preferred integrated coherent baseline |
| DQPSK | 2 bits/symbol | ≈100 GBd | More efficient than binary formats; receiver architecture differs from coherent QAM |
| Single-polarization 16-QAM | 4 bits/symbol | ≈50 GBd | Good spectral efficiency, but one polarization alone does not directly provide 200 Gb/s at 50 GBd |
| DP-16QAM | 4 bits/symbol × 2 polarizations | 200 / (4 × 2) = 25 GBd idealised | Strong fit on paper; practical line rate must include FEC/framing and pulse-shaping overhead |

The idealized baud-rate equation is R_s = R_b / (m × N_pol), where R_b is bit rate, m is bits per symbol per polarization and N_pol is the number of independently used polarizations. For DP-16QAM, m = log2(16) = 4 and N_pol = 2, so R_s = 200 Gb/s / 8 = 25 GBd under the simplified net-rate assumption. Actual commercial transceivers may use a higher line rate to accommodate FEC and framing overhead.

For PAM4, two bits are represented by each symbol, so a 200 Gb/s single-lane stream requires approximately 100 GBd before overhead. PAM4 is not directly comparable to DP-16QAM because the latter uses two polarizations and coherent phase/amplitude detection. The comparison is therefore based on an idealized information-rate calculation, not equal hardware complexity.

## Spectral efficiency and channel spacing:

For an idealized 200 Gb/s DP-16QAM channel on a 50 GHz grid, the gross grid-based efficiency is 200/50 = 4 bit/s/Hz. This is a planning ratio; true spectral efficiency depends on net payload, pulse-shaping roll-off, guard bands and the occupied spectrum.

A simplified Nyquist bandwidth estimate is B ≈ R_s(1 + α), where α is the pulse-shaping roll-off factor. With R_s = 25 GBd and an assumed α = 0.1, B ≈ 27.5 GHz. A nominal 50 GHz grid leaves room for filtering and guard band in this idealized case. The actual transmitter spectrum and receiver filter response must be checked in simulation.

For 100 GBd PAM4 and the same α = 0.1, the estimate is about 110 GHz. This would not fit cleanly into a 50 GHz slot without a different signalling or multi-lane interpretation. This simple calculation highlights why the coherent DP-16QAM option is attractive for a 200 Gb/s channel on a conventional DWDM grid.

## Modulation mechanism comparison:

| Mechanism | Speed / response | Vπ and chirp considerations | Suitability |
| --- | --- | --- | --- |
| Electro-optic (EO) | Can support high-speed data modulation when device and driver are designed appropriately | Vπ depends on material, electrode geometry and length; MZI can be designed for low chirp or controlled chirp | Preferred for data-rate modulation |
| Acousto-optic | Uses an acoustic wave to change optical properties; response is generally limited by acoustic transit time | Not normally specified by a data-modulator Vπ; comparatively slow for very high baud rates | Not selected for 200 Gb/s symbol modulation |
| Thermo-optic | Changes refractive index through heating; response is typically slow | Power is used for heating and tuning; not a high-speed data mechanism | Useful for tuning/locking rings and filters |
| Magneto-optic | Uses magnetic-field-dependent optical properties | Implementation and integration complexity can be high | More relevant to polarization/isolating functions than the data modulator |
| Kerr effect | Intensity-dependent refractive index; response is intrinsically fast | Nonlinear phase depends on optical intensity and material; can be useful but requires careful control | Not selected as the main transmitter modulator |

A single universal Vπ or bandwidth value cannot be assigned to a material family: the value depends on the actual device geometry and datasheet. For this reason, the design requirement is to select a qualified high-speed EO modulator whose measured electro-optic bandwidth covers the selected baud rate with margin, and then record its measured Vπ, insertion loss and chirp in the final parameter sheet.

## MZI versus micro-ring resonator:

| Feature | Silicon-photonic MZI | Micro-ring resonator (MRR) | EO-polymer implementation |
| --- | --- | --- | --- |
| Footprint | Moderate; longer interferometer arms | Very compact resonant structure | Can be integrated with compact EO devices; geometry-dependent |
| Bandwidth | Broadband operation is possible with suitable electrodes | Resonance limits usable bandwidth and requires wavelength alignment | Potentially high EO efficiency; verify device-specific bandwidth |
| Thermal sensitivity | Phase drift can occur but can be controlled | Strong wavelength sensitivity; often needs thermal tuning | Material and packaging stability must be qualified |
| Insertion loss | Splitter/combiner and waveguide losses contribute | Coupling and resonant loss can be significant | Depends on waveguide, electrodes and polymer loss |

## Design Decision :

I recommend DP-16QAM using a high-speed electro-optic silicon-photonic Mach–Zehnder modulator because it provides an idealized 25 GBd rate for 200 Gb/s under the stated simplified assumption and gives coherent DSP access to phase and amplitude information. A 50 GHz channel grid is a reasonable starting point for this baud-rate estimate. The selection is conditional on measured bandwidth, Vπ, insertion loss, OSNR requirement and pre-FEC BER being verified for the chosen transceiver. A micro-ring design could reduce footprint but introduces tighter resonance and thermal-control requirements.

# Optical Amplification Design

## Amplifier comparison

| Amplifier | Advantages | Constraints | Role in proposed system |
| --- | --- | --- | --- |
| EDFA | Mature technology, useful C-band gain and output power | Adds ASE; gain tilt and noise accumulate across spans | Lumped gain and output power |
| SOA | Compact and potentially integrable | Nonlinearities, polarization sensitivity and inter-channel effects can be important | Not the primary line amplifier for this high-capacity baseline |
| Lumped Raman | Can provide useful gain and spectral flexibility | Pump power and noise/control design require attention | Alternative where span-specific benefit is demonstrated |
| Distributed Raman | Gain is distributed along transmission fibre and can improve effective noise performance | Pump complexity, safety and operational control | Improves span OSNR in the hybrid design |
| Hybrid Raman + EDFA | Combines distributed gain with practical EDFA output level | Higher cost, control and monitoring complexity | Selected, provided the link budget justifies the added hardware |

## Illustrative amplifier power calculation:

The assignment asks for input and target output power per channel but the numeric levels are missing in the supplied brief. To demonstrate the method, assume an amplifier receives −10 dBm per channel and the target is 0 dBm per channel. These are calculation assumptions, not values supplied by the assignment.

Per-channel span gain = P_out − P_in = 0 − (−10) = 10 dB.

Total output power for 32 equal-power channels is P_total(dBm) = P_channel(dBm) + 10 log10(N).

P_total = 0 + 10 log10(32) = 15.05 dBm.

In watts, 0 dBm = 1 mW per channel, so total power = 32 × 1 mW = 32 mW = 0.032 W. Converting 15.05 dBm to watts also gives approximately 0.032 W.

If the chosen target were +3 dBm per channel instead, total output would be 18.05 dBm, or about 63.8 mW, and the gain from −10 dBm would be 13 dB. The final value must be recalculated using the actual transceiver launch power, span loss, connector loss and required amplifier output.

## Gain flatness and ASE noise:

A DWDM amplifier rarely has perfectly equal gain at every wavelength. Gain tilt causes some channels to become stronger than others after several spans. A gain-flattening filter (GFF), wavelength-aware dynamic gain equalization and per-channel monitoring can reduce this imbalance. The design should include adequate amplifier saturation margin so that adding or removing channels does not cause unacceptable power changes.

Amplified spontaneous emission (ASE) is added at each amplifier. Across cascaded spans, the accumulated noise reduces OSNR even if the channel power is restored after every span. A proper OSNR budget must use the amplifier noise figure, gain, optical reference bandwidth, span count and fibre loss. Those parameters are not fully specified in the assignment, so a numerical end-to-end OSNR cannot be truthfully calculated yet.

## Design Decision :

I recommend a hybrid distributed Raman plus EDFA arrangement for a long-reach design candidate because distributed gain can improve the effective noise performance while the EDFA provides controlled lumped gain. The added complexity is justified only if link-level simulation shows a useful OSNR or reach improvement compared with EDFA-only amplification. The assumed example requires 10 dB gain per span and 15.05 dBm aggregate output for 32 channels at 0 dBm each. GFF and per-channel equalization should be included.

Impairment Mitigation and Coherent DSP

## Physical impairment mechanisms:

Chromatic dispersion spreads pulses because optical frequencies travel at different group velocities. PMD arises when fibre birefringence causes different delays for orthogonal polarization components. The Kerr effect makes refractive index intensity-dependent and produces SPM, XPM and FWM in multi-channel transmission. PDL creates polarization-dependent attenuation, while mux/demux imperfections can introduce crosstalk and passband narrowing.

## Optical compensation versus DSP

| Method | Benefits | Trade-offs |
| --- | --- | --- |
| Dispersion-compensating fibre (DCF) | Can compensate accumulated chromatic dispersion in the optical domain | Adds loss, footprint and nonlinear interaction; may be unnecessary in coherent systems |
| Optical phase conjugation | Can compensate selected propagation impairments when the optical link is suitably arranged | Requires careful symmetry and hardware; not a universal solution |
| Coherent DSP CD equalization | Flexible, tunable and avoids a separate DCF module | Consumes digital processing power and needs accurate receiver implementation |
| Adaptive MIMO equalizer | Tracks polarization rotation and PMD | Tap count and convergence depend on baud rate and channel conditions |
| Digital backpropagation (DBP) | Can partly reverse modelled nonlinear propagation | Computationally intensive; gains depend on model accuracy, span count and implementation |

## Recommended receiver DSP pipeline:

Optical front end: coherent integrated receiver converts the received optical field into electrical I/Q signals for both polarizations.

Analog front-end conditioning and ADC: sample the electrical waveforms at a rate appropriate to the selected baud rate and receiver design.

Deskew, resampling and timing recovery: align streams and establish symbol timing.

Chromatic-dispersion compensation: use frequency-domain equalization or an equivalent digital filter to remove the dominant linear dispersion.

Adaptive polarization demultiplexing and PMD compensation: use a dual-polarization MIMO equalizer; CMA can support initial blind convergence where appropriate.

Frequency-offset estimation and carrier-phase recovery: correct laser frequency difference and track phase noise, particularly important for QAM.

Symbol demapping and soft-decision generation: estimate transmitted bits and reliability information.

FEC decoding and performance monitoring: record pre-FEC BER, post-FEC BER, estimated Q-factor and margin to the selected threshold.

Optional DBP: enable only if simulations show that nonlinear compensation produces enough performance gain to justify power and computational cost.

The precise ordering can vary by implementation. Timing recovery, resampling, dispersion equalization and adaptive equalization may interact, so the final ordering and algorithm settings must be validated against the chosen receiver and modulation format.

## DBP trade-off

DBP uses a numerical inverse-propagation approach, commonly based on the split-step Fourier method. It can mitigate deterministic nonlinear distortion when the fibre and amplifier model is sufficiently accurate. However, its computational burden increases with the number of steps, channels modelled and sample rate. A practical deployment should first use linear coherent DSP and optimized launch power; DBP should be added only if the Q-factor or reach gain is demonstrated by simulation.

## Design Decision

I recommend coherent DSP with CD equalization, adaptive dual-polarization MIMO equalization, carrier-frequency estimation, carrier-phase estimation and FEC. This sequence directly addresses the major linear impairments and coherent demodulation needs. DBP is an optional advanced block rather than a mandatory baseline because its energy and processing cost may outweigh its benefit. The design must report actual Q-factor and BER from simulation instead of assuming a numerical improvement.

# Integrated DWDM Architecture

# Block-level system diagram

32 narrow-linewidth lasers ↓ DP-16QAM ↓ Per-channel ↓ AWG ↓ SMF ↓ EDFA ↓ AWG ↓ Dual-polarization ↓ ADC ↓ frequency/phase recovery

Proposed block-level architecture. Faraday rotators and polarization-management elements may be used where required by the optical design, but they do not replace coherent polarization demultiplexing. Circulators are included only where the Raman-pump or optical path design requires them.

## System parameter table

| Parameter | Proposed baseline / value | Status |
| --- | --- | --- |
| DWDM channels | 32 | Given |
| Per-channel rate | 200 Gb/s | Given; clarify payload versus line rate |
| Aggregate rate | 6.4 Tb/s | Calculated |
| Optical band | C-band, approximately 1530–1565 nm | Design assumption |
| Frequency grid | 50 GHz nominal | Design assumption; confirm applicable grid/interface |
| Modulation | DP-16QAM | Selected candidate |
| Idealized symbol rate | 25 GBd at 200 Gb/s net and no overhead | Calculated; practical rate depends on overhead |
| Pulse-shaping roll-off | 0.1 used for illustrative bandwidth estimate | Assumption |
| Modulator | High-speed EO silicon-photonic MZI | Selected candidate; verify datasheet |
| Laser linewidth | Use qualified coherent-transceiver specification; set in simulation to the actual selected laser value | Must be filled from component datasheet |
| Modulator Vπ | Use measured/datasheet Vπ for the chosen MZI | Must be filled from component datasheet |
| Amplification | Distributed Raman + EDFA, GFF and power equalization | Selected candidate |
| Example amplifier gain | 10 dB for −10 to 0 dBm/channel | Illustrative assumption |
| Example total amplifier output | 15.05 dBm ≈ 32 mW for 32 channels at 0 dBm/channel | Calculated |
| Fibre attenuation | Use measured/specification value; representative standard SMF is often modelled near 0.2 dB/km around 1550 nm | Simulation input must be confirmed |
| Dispersion coefficient | Use fibre datasheet; representative C-band SMF value is around 17 ps/(nm·km) | Simulation input must be confirmed |
| PMD coefficient | Use fibre datasheet or a stated statistical model | Must be specified |
| DSP filter taps | Choose by baud rate, dispersion, PMD and convergence testing; sweep in simulation | To be optimized |

## Couplers, mux/demux and polarization components

AWGs are suitable for wavelength-selective multiplexing and demultiplexing. MMI couplers and directional couplers split or combine optical power according to their geometry and coupling behaviour. Every passive component adds insertion loss and may contribute crosstalk. The transmitter launch-power setting must therefore be calculated after accounting for modulator, coupler, mux and connector losses. Optical isolators reduce unwanted back-reflections; circulators route light between ports where required. Polarization rotators or controllers may support the optical path, while the coherent receiver DSP handles changing state of polarization.

## Integrated trade-offs and failure points

| Issue | Risk | Control / evidence required |
| --- | --- | --- |
| High-order modulation | OSNR shortfall causes BER rise | OSNR sweep and pre-FEC BER/Q curves |
| Amplifier gain tilt | Unequal channel powers after multiple spans | GFF, dynamic equalization and per-channel monitoring |
| Excess launch power | SPM/XPM/FWM reduce quality | Sweep launch power per channel and choose optimum |
| Filter narrowing / crosstalk | Passband clipping and adjacent-channel leakage | Optical spectrum and filter-cascade simulation |
| Thermal drift in photonics | Wavelength or phase changes | Thermal control, locking and environmental tests |
| DSP power and latency | High implementation power or delay | Measure computational load; enable DBP only if justified |
| Standards mismatch | Interoperability failure | Map each interface to the exact ITU-T / IEEE / OIF specification and test limits |

# Verification and Critical Evaluation

The table below separates values that can be calculated from the assignment from values that still require component specifications or simulation. This avoids presenting assumed performance as measured evidence.

| Requirement / parameter | Target / design value | Evidence status |
| --- | --- | --- |
| Aggregate capacity | 6.4 Tb/s = 32 × 200 Gb/s | Analytically verified |
| Spectral efficiency | 4 bit/s/Hz gross grid ratio for 200 Gb/s on 50 GHz | Calculated planning ratio; confirm net payload and occupied spectrum |
| Modulation | DP-16QAM | Selected; validate OSNR and pre-FEC BER |
| Idealized baud rate | 25 GBd DP-16QAM; 100 GBd PAM4 comparison | Calculated before overhead |
| Modulator bandwidth / footprint | Bandwidth must cover actual baud rate with margin; compact PIC preferred | Datasheet and layout evidence required |
| Amplifier gain | 10 dB in illustrative −10 to 0 dBm/channel example | Calculated under stated assumptions |
| Aggregate amplifier output | 15.05 dBm / 0.032 W for 32 × 0 dBm | Calculated under stated assumptions |
| Gain flatness | Within the selected amplifier/system specification | Needs GFF and channel-power simulation |
| OSNR / pre-FEC BER | Above chosen transceiver/FEC operating threshold with engineering margin | Not yet measured; simulation required |
| CD / PMD mitigation | DSP equalization and adaptive MIMO | Architecture selected; verify using fibre model |
| Nonlinear tolerance | Optimized launch power; optional DBP | Requires Q/BER versus launch-power sweep |
| Standards | Applicable ITU-T G.698 interface and relevant IEEE/OIF interface requirements | Exact interface variant must be identified and checked |
| Deployment readiness | Prototype first, field deployment after verification | Conditional |

## How performance should be evaluated

OSNR margin: compare simulated OSNR at the coherent receiver with the specified requirement for the selected modulation and FEC mode.

BER: report both pre-FEC and post-FEC BER. Do not call a link error-free solely because the pre-FEC BER is below an assumed value.

Q-factor: state the simulator's definition and report the minimum value across all 32 channels.

Spectral efficiency: calculate using net delivered bit rate divided by occupied spectrum, not only the channel-grid ratio.

Power per bit: measure total electrical power of lasers, drivers, modulators, pumps, amplifiers, ADCs/DSP and cooling, then divide by delivered bit rate.

Footprint: estimate transmitter PIC, amplifier modules, mux/demux, receiver DSP and thermal-control packaging from selected component data.

## Standards and compliance note

The brief names ITU-T G.698, IEEE 802.3ct/ck and OIF-CEI. These documents cover different interface layers and use cases. A design review should identify the exact recommendation or standard revision applicable to the selected optical reach, wavelength plan, client interface and electrical lane. Compliance cannot be confirmed from the architecture alone; it requires checking the specified optical power, wavelength/frequency, transmitter quality, receiver sensitivity, jitter, electrical channel and interoperability limits applicable to the selected interface

Simulation using MATLAB:

CODE:

clc;

clear;

close all;

Nch = 32;

Rb = 200e9;

gridGHz = 50;

rolloff = 0.1;

totalCapacity = Nch * Rb;

baud16QAM = Rb / 8;

baudPAM4 = Rb / 2;

bandwidth16QAM = baud16QAM * (1 + rolloff);

fprintf('ULTRA-HIGH-SPEED PHOTONIC TRANSPORT LINK\n');

fprintf('----------------------------------------\n');

fprintf('Number of DWDM channels = %d\n', Nch);

fprintf('Data rate per channel = %.0f Gb/s\n', Rb/1e9);

fprintf('Total capacity = %.2f Tb/s\n', totalCapacity/1e12);

fprintf('DP-16QAM symbol rate = %.2f GBd\n', baud16QAM/1e9);

fprintf('PAM4 symbol rate = %.2f GBd\n', baudPAM4/1e9);

fprintf('DP-16QAM bandwidth = %.2f GHz\n', bandwidth16QAM/1e9);

inputPower_dBm = -10;

outputPower_dBm = 0;

gain_dB = outputPower_dBm - inputPower_dBm;

totalPower_mW = Nch * 10^(outputPower_dBm/10);

totalPower_W = totalPower_mW / 1000;

fprintf('\nAMPLIFIER ANALYSIS\n');

fprintf('Amplifier gain = %.2f dB\n', gain_dB);

fprintf('Total output power = %.4f W\n', totalPower_W);

frequency = linspace(193.1e12,193.1e12+31*gridGHz*1e9,2000);

spectrum = zeros(size(frequency));

for k = 1:Nch

fc = 193.1e12 + (k-1)*gridGHz*1e9;

spectrum = spectrum + exp(-((frequency-fc)/(10e9)).^2);

end

figure;

plot((frequency-193.1e12)/1e9,spectrum,'LineWidth',1.5);

xlabel('Frequency Offset (GHz)');

ylabel('Normalized Optical Power');

title('32-Channel DWDM Optical Spectrum');

grid on;

launchPower = -10:1:5;

Qfactor = 9.5 - 0.20*(launchPower+2).^2;

Qfactor(Qfactor < 0) = 0;

figure;

plot(launchPower,Qfactor,'o-','LineWidth',1.5);

xlabel('Launch Power (dBm)');

ylabel('Illustrative Q-Factor (dB)');

title('Q-Factor versus Launch Power');

grid on;

OSNR = 5:1:25;

BER = 0.5*erfc(sqrt(10.^(OSNR/10)));

figure;

semilogy(OSNR,BER,'s-','LineWidth',1.5);

xlabel('OSNR (dB)');

ylabel('Approximate BER');

title('Illustrative BER versus OSNR');

grid on;

Modulation = {'DP-16QAM'; 'PAM4'};

SymbolRate_GBd = [baud16QAM; baudPAM4]/1e9;

EstimatedBandwidth_GHz = [bandwidth16QAM; baudPAM4*(1+rolloff)/1e9];

Results = table(Modulation,SymbolRate_GBd,EstimatedBandwidth_GHz);

disp(Results);

OUTPUTS:

# Conclusion and Future Work

This investigation proposes a 32-channel, 6.4 Tb/s DWDM link based on DP-16QAM coherent transmission, high-speed electro-optic silicon-photonic MZI modulators, hybrid Raman–EDFA amplification and coherent DSP. The idealized rate calculation gives 25 GBd for a 200 Gb/s DP-16QAM channel before implementation overhead, compared with about 100 GBd for a single-stream PAM4 example. A 50 GHz grid is a plausible planning baseline for the coherent design, subject to pulse-shape and interface verification.

The amplifier example demonstrates the required calculation method: 10 dB gain moves a channel from −10 to 0 dBm, and 32 channels at 0 dBm each produce approximately 15.05 dBm or 32 mW aggregate optical output. This result depends on the assumed power levels. Hybrid amplification, gain flattening and channel equalization are recommended for evaluation, while DSP provides the primary flexible approach to linear impairment compensation.

Future work should use actual component datasheets and a complete fibre route to build the OptiSystem/OptSim/MATLAB model. The next step is to generate optical spectra, constellations, BER-versus-OSNR and Q-factor-versus-launch-power plots, compare amplifier options, optimize DSP settings and complete standards-based interoperability testing. The system should be deployed only when measured or simulated evidence confirms sufficient margins.

# References

- Agrawal, G. P. (2021). *Fiber-optic communication systems* (5th ed.). Wiley.

- International Telecommunication Union. (n.d.). *ITU-T G.698 series: Multichannel optical interfaces for wavelength division multiplexing applications*. ITU-T. Verify the applicable in-force recommendation and revision for the selected interface.

- Institute of Electrical and Electronics Engineers. (2022). *IEEE Std 802.3ck-2022: IEEE standard for Ethernet—Amendment: Media access control parameters, physical layers, and management parameters for 100 Gb/s, 200 Gb/s, and 400 Gb/s electrical interfaces*. IEEE.

- Institute of Electrical and Electronics Engineers. (2020). *IEEE Std 802.3ct-2020: IEEE standard for Ethernet—Media access control parameters, physical layers, and management parameters for 100 Gb/s operation over DWDM systems*. IEEE.

- Optical Internetworking Forum. (n.d.). *Common Electrical I/O (CEI) implementation agreements*. OIF. Consult the applicable CEI version for the chosen electrical interface.

- Essiambre, R.-J., & Tkach, R. W. (2012). Capacity trends and limits of optical communication networks. *Proceedings of the IEEE, 100*(5), 1035–1055. https://doi.org/10.1109/JPROC.2012.2182970

- Winzer, P. J., & Neilson, D. T. (2017). From scaling disparities to integrated parallelism: A decathlon for a decade. *Journal of Lightwave Technology, 35*(5), 1099–1115. https://doi.org/10.1109/JLT.2017.2662082

- Essiambre, R.-J., Kramer, G., Winzer, P. J., Foschini, G. J., & Goebel, B. (2010). Capacity limits of optical fiber networks. *Journal of Lightwave Technology, 28*(4), 662–701. https://doi.org/10.1109/JLT.2009.2039464

- Secondini, M., & Forestieri, E. (2013). Scope and limitations of the nonlinear Shannon limit. *Journal of Lightwave Technology, 35*(4), 893–902. Check bibliographic details against the publisher record before final submission.

- Note: Standards and journal references should be checked against the institution's required citation style and the latest official revision before submission.